BEFORE BOARDING

# DTA TRIKE CHECKLISTS

Parking brake – ON

Chocks – REMOVED

Gas cap – SECURE

Saddle bags – ZIPPED

## PASSENGER BRIEFING

Loose items secure

CRM/Passenger participation

Collision avoidance

Explanation of throttle, steering, training bars – DO NOT OVERRIDE
Where to put hands

Lost communication signals

Airsickness

Helmet strap instructions

Seat belt instructions

“Light-Sport…”

## BEFORE START

Seatbelts – ON

Helmets straps – ON

Parachute - ARM

Radio, Transponder, GPS – OFF

Throttle 1, 2, 3 – CLOSED

Choke – ON/OFF

Master – ON

Brake - ON

START

## AFTER START

Oil Pressure – Check

Choke – OFF

Radio - ON, Transponder – STBY

Nav Lights – ON

Foot brake – CHECK BEFORE TAXI

## BEFORE TAKEOFF

Controls – CHECK

Radio frequency – SET 
Transponder - ALT

Altimeter – SET

Runup, Engine temps – OT 120degF

Mags 1, 2 – TEST

Fuel quantity – CHECK

Helmet visors – DOWN

Traffic – CHECK

After takeoff – CALL OUT 350’ agl

## BEFORE LANDING

Altimeter – SET

Fuel quantity – CHECK

Nosewheel - STRAIGHT

Brakes – OFF

Controls – PAX CLEAR

## AFTER LANDING

Wing – SECURE

Radio frequency – SET

## SHUTDOWN

Radio, Transponder, GPS – OFF

Master – OFF

All switches - OFF

Ignition – OFF

Flight time - NOTE

Parachute – SAFETY ON

## IN FLIGHT SHUTDOWN

Engine – COOL @ REDUCED RPM

Aircraft traffic – CHECK

Radio, Transponder, GPS – OFF

Mags – OFF

Radio – ON

## IN FLIGHT RESTART

Radio – OFF

Mags – ON

Engine – START

Radio, Transponder, GPS – ON

Engine temperatures – CHECK

## ENGINE FIRE

Mags – OFF

Radio – DECLARE EMERGENCY

Master – OFF

Fire Extinguisher – ACTIVATE